
PERFORMANCE SPECIFICATION

	2.2     Transceiver Terminal

	2.2.1Audio / Video interface

i. The unit shall have BNC SD/HD SDI output with embedded audio.

ii. The unit shall have BNC SD/HD SDI input for ingest.

iii. It shall support 
unlock for Tri-level or black burst.

	2.2.2 IFB Input

Support external USB audio input with level control (mic/line) or XLR.

	2.2.3Local storage

It shall support storing of content in its on-board storage and output it over SDI audio video.

	2.2.4Configuration

It shall come in standard 19” 1RU rack mount or workstation.

	2.2.5Network I/O

It shall have 10/100/1000 BASE-T RJ45 Ethernet Interfaces and USB2.0/3.0.

	2.2.6KVM


The unit shall come with KVM extender of keyboard/mouse and video to operation room.

	2.2.7Monitoring And Control


The unit shall have monitoring feature for the incoming feed, bandwidth, data rate and provide complete configuration control over transmitter system in the field.

	2.2.8 Other Functionality
The unit shall support ingest of SD/HD SDI signal from local feeds and simultaneously distribute the live feed to other remote transceivers through internet.



