| Sr. No | Description | Model | Make | Qty |
| --- | --- | --- | --- | --- |
| 1 | Differential Pressure Switches LDD: Differential pressure control, two independently adjustable set points, fixed dead band N4: NEMA 4, 4 X L-Series: Epoxy coated, Dei Cast Aluminum, IP66 KK: Dual Switch Narrow Dead band Actuator Seal: Buna-N (-18….65oC) (B) 1/4 NPT Female and ½ NPT Male Combination pressure only XFS: Factory adjusted set point TM: 2" pipe mounting bracket Range: 0–4 Bar Set Point: 380 mbar decreasing, 2 bar increasing | LDD-N4-KK-B-06-XFS-TM-0/4Bar or Equivalent | Ashcroft, USA Or Equivalent | 140 |