| Item # | Item Description | Unit of Measure | Q-ty | Price in USD or UZS (exclusive of VAT) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Unit Price | Total Price |
| 1 | FODDER SHREDDER Capacity: min. 300 kg/h Electric motor capacity: max 5,5 кW Electric motor type: 3 phases Electric motor voltage and frequency of current: 380 V, 50 Hz Replaceable circular sieve: with 3 mm and 8 mm holes Overall dimensions of the drum: up to 500x500x100 mm Overall dimensions: up to 650x500x1000 mm | Units | 6 |  |  |
| 2 | FEED MIXER Capacity: min. 300 kg/h Working capacity of the mixer: 100-150 kg / loading Electric motor capacity: max 1,5 кW Electric motor type: 3 phases Electric motor voltage: 380 V Duration of mixing: 5 to 8 minutes The height of the discharge pipe above floor level: 200-280 mm Overall dimensions of the bunker: up to 850x850x580 mm Overall dimensions: up to 850x850x1100 mm | units | 6 |  |  |
|  | FEED PELLETIZER Capacity: min. 300 kg/h Electric motor capacity: max 12 кW Electric motor type: 3 phases Electric motor voltage and frequency of current: 380 V, 50 Hz The diameter of the radial holes in the matrix: 3-8 mm Overall dimensions: up to 1500x400x1150 mm Matrix characteristics: -Hardness of dies and rollers on Rockwell C Hardness scale: 55-58 HRC - test loading: 150 kgf - type of indenter: 120 ° diamond, sphero-conical | Units | 6 |  |  |
|  | A PROTECTION AND CONTROL UNIT should provide protection for electric motors: • from multi-phase and single-phase short circuits; • from phase failure (from work on 2 phases); • from the wrong phase rotation; • against overvoltage; • against undervoltage; • from short to ground of one of the phases; • from thermal overload. The composition of the block of protection and control should include: • Voltmeter for each phase (with LCD display); • Ammeter for each electric motor (with LCD display); • Thermo-Hygrometer (with LCD display); • voltage monitoring relay; • Phase control relay; • Overcurrent control relay; • Electromagnetic contactor; • Automatic modular switch; • Automatic power switch; • Inverter frequency; • "Start" button; • "Stop" button; • Emergency Stop button; • Indicator lamp; • Clamps, terminals, DIN-rail; • Electrical cable; • Mounting shield. | Units | 6 |  |  |
|  | Total Prices of Goods |  |  |  |  |
|  | Transportation costs |  |  |  |  |
|  | Packing, Insurance and other costs |  |  |  |  |
|  | Training |  |  |  |  |
|  | Installation and commissioning |  |  |  |  |
|  | Total All-Inclusive Bid Price |  |  |  |  |

Notes:

- The bidders must quote genuine, branded products. UNDP may conduct post-delivery inspection at its sole discretions.

- Sample of all items to be submitted with the bid response (as applicable).

- Price to include delivery, installation/ testing/ commissioning (as applicable)

- Site survey will be arranged on request (as applicable)

- Brochures and Technical details of the model offered MUST be submitted with the bid offer (as applicable)