Basic Data for Design of the Plant

  - Atmospheric design data
Atmospheric pressure	98kPa（A）
Maximum temperature	45 C
Minimum temperature	-5C

Relative humidity	80%
Oxygen content in atmospheric air	20.95 %O2
Sea level elevation	～100 meter

  - Circulating water
Inlet temperature	≤35℃
Inlet pressure	0.3~0.4 MPa（G）
Backwater pressure	0.15~0.2 MPa（G）
Fouling factor	0.000344m2·℃/Wl

PH value                                 7.8～8.3

Hardness                                 ≤5mg/L

  - Power supply

High voltage: 3,000V  10 %, AC   (3-phase, 3-wire, unearthed neutral)
Low voltage: 400V/220V  10%, AC (3-phase, 4-wire, directly earthed neutral)	
Frequency	50Hz

1.5      Instrument air (used in start-up) to be supplied by the client
         Pressure	0.6MPa（G）
         Flowrate	～180 Nm3/h
         Temperature	≤40℃

Dust content	≤3 μm
Dew point	-40℃

Roots Blower

Specifications of Roots Blower

| media | Air | Remarks |
| --- | --- | --- |
| Name of equipment | Roots Blower | GardnerDenver-USA |
| Type | Sutorbilt 8000Q-2242 |  |
| Inlet pressure | 0KPa（g） |  |
| Inlet temperature | 0-40℃ |  |
| Inlet flow | 486m3/min |  |
| Extreme outlet pressure | 49KPa（g） |  |
| Rotation | 740rpm |  |
| Outlet temperature | 72℃ |  |
| Shaft power | 530.4KW |  |
| Motor power | 630KW/3.0KV | Protection level: IP23 Insulation level: F |
| Cooling method | Natural air cooling |  |
| Driving method | Coupler |  |
| Working policy | Continuously or discontinuously |  |

3.4.2 Roots Vacuum Pump

Specifications of Roots Vacuum Pump

| media | Air | Remarks |
| --- | --- | --- |
| Name of equipment | Roots Vacuum Pump | GardnerDenver-USA |
| Type | Sutorbilt 8000Q-2254 |  |
| Inlet pressure | 0KPa（g） |  |
| Inlet temperature | 0-40℃ |  |
| Inlet flow | 602m3/min |  |
| Extreme outlet pressure | -50KPa（g） |  |
| Rotation | 740rpm |  |
| Outlet temperature | 127℃ |  |
| Shaft power | 658.7KW |  |
| Motor power | 800KW/3.0KV | Protection level: IP23 Insulation level: F |
| Cooling method | Natural air cooling |  |
| Driving method | Coupler |  |
| Working policy | Continuously or discontinuously |  |