| Sr# | Description | Qty |
| --- | --- | --- |
| 1 | Liquid Ring Compressor - Gas Handled: varying composition of Hydrocarbons and Nitrogen, Ethylene Dichloride (EDC), Benzene, Chloroform, Carbon Tetrachloride - Molecular Weight: 99 (for design basis) - Suction Pressure: 1.5 kPa (g) - Discharge Pressure: 70 kPa (g) - Suction Temperature: 70 deg C - Discharge Temperature: vendor to specify - Capacity/Flow: 560 m3/hr (Max) (the unit should be able to operate on total recycle also) - Sealing Fluid: Water - Preferred Material of Construction: Vendor to specify (should be compatible with the gases handled and traces of HCl) | One |