VENDORS MUST INDICATE COMPLIANCE OF THE GOODS OFFERED WITH THE FOLLOWING TECHNICAL SPECIFICATIONS.

Item 1 - Digging Hoe

| # TECHNICAL SPECIFICATIONS REQUIRED |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |
| --- | --- | --- | --- | --- |
|  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |
| Item 1 |  | Description and purpose of the item | Make/ brand and model of item |  |
| Digging Hoe |  | Rectangular blade, heavy weight, digging hoes in alloy steel, without handle. |  |  |
| Unit |  | Hoe blade |  |  |
| Total quantity of units required |  | 51,990 Pieces | Name and address of the manufacturer of item |  |
| Country |  | Project code |  |  |
| Sudan |  | - 3,000 pieces Under FAO Project OSRO/SUD/809/ITA; - 8,990 pieces Under 2019 OFDA Darfur ; - 24,000 pieces Under 2019 CERF Proposal ; - 5,500 Pieces Under 2019 UNTFHS Proposal ; - 9,000 pieces Under 2019 Korean Proposal ; - 1,500 pieces Under 2019 ECHO-Allait Proposal ; |  |  |
| 1.0. | Technical Specifications: General |  |  |  |
| 1.1 | Fitting for handle: sunken eye |  |  |  |
| 1.2 | Without handle |  |  |  |
| 1.3 | Length from the top of blade to the cutting edge, excluding eye (mm): Approx. 160 |  |  |  |
| 1.4 | Blade Width (mm): Approx. 140 |  |  |  |
| 1.5 | Blade Thickness (mm): 2.5 – 3.0 |  |  |  |
| 1.6 | Central reinforcement of the blade starting from the eye up to mid of the blade |  |  |  |
| 1.7 | Blade Weight (kg): Approx. 1.5 |  |  |  |
| 1.8 | Configuration of the eye (mm): Oval, 50 x 25 approx. |  |  |  |
| 1.9 | Thickness of material around the eye to range from a minimum of 8 mm at widest point to a minimum of 5 mm at narrowest. |  |  |  |
| 2.0 | Technical Specifications: Material specifications |  | IMPORTANT: FAO reserves the right to apply penalties after delivery if the lab test on material specifications show results that differ from what the bidder confirmed to comply |  |
|  | Blade: The steel for agricultural hoes shall be a medium to high carbon steel within the specified ranges given below. |  |  |  |
| 2.1 | Carbon : 0.40 – 0.80 % |  |  |  |
| 2.2 | Manganese : 0.50 – 0.90 % |  |  |  |
| 2.3 | Silicon : 0.35 % max. |  |  |  |
| 2.4 | Phosphorous : 0.06 % max. |  |  |  |
| 2.5 | Sulfur: 0.06 % max. |  |  |  |
| 3.0 | Technical Specifications: Heat treatment and hardness |  |  |  |
|  | After the forging has been normalised, the lower part of the blade adjacent to the cutting edge shall be hardened and tempered to give the following conditions: |  |  |  |
| 3.1 | Area immediately adjacent to eye: Rockwell C 25/30. |  |  |  |
| 3.2 | Hardness within 50 mm of the cutting edge: Rockwell C 40/50. |  |  |  |
| 3.3 | This hardened zone shall not extend more than half way towards the tang or eye and shall gradually decrease to the normalised zone around the eye. |  |  |  |
| 4.0 | Technical Specifications: Construction |  |  |  |
| 4.1 | Blade: The forging must be symmetrical and free from flaws. All fins and flashes must be dressed off. The eye must be smooth internally, uniformly tapered, and must lie centrally in the forging. |  |  |  |
| 4.2 | Blade: The cutting edge must be ground sharp, preferably with an included angle of 18 to 23° and shall be ground on the front side of the blade, i.e. the side facing the operator. |  |  |  |
| 4.3 | Blade: For digging hoes, the rise of the blade shall be small (preferably less than 55 mm) so that the blade is almost at right angle to the handle to give maximum penetration on impact. |  |  |  |
| 5.0 | Technical Specifications: Marking |  |  |  |
|  | The forging should be clearly and indelibly marked with the following: |  |  |  |
| 5.1 | Manufacturers name and/or trademark. |  |  |  |
| 5.2 | The nominal weight or size of the head. |  |  |  |
| 5.3 | The pattern number. |  |  |  |
| 6.0 | Technical Specifications: Preservative Treatment |  |  |  |
| 6.1 | The heads to be varnished or painted all over |  |  |  |
| 6.2 | If they are to be shipped by sea, then they should also be greased or individually wrapped in a rust inhibiting covering. |  |  |  |
| 6.3 | They must also be packed in strong Wooden boxes of 24 pieces with metallic belt |  |  |  |
| 7.0 | ##### Technical Specifications: Strength testing |  | ##### IMPORTANT: FAO reserves the right to apply penalties if the strength tests conducted before load and after delivery show results that differ from what the bidder confirmed to comply |  |
|  | ##### In addition to meeting the above specifications, the tools shall also meet the following strength tests: |  | ##### State below whether the tools offered will be able to meet the Strength Tests listed. |  |
| 7.1 | Strength Test: The tool is fitted with a temporary handle made from a metal tube and clamped as shown in the figure. A load of 30 kg is applied in increments of 5 kg., suspended from the handle at a distance of 1000 mm from the eye (the handle may not necessarily be horizontal). On removal from the eye, the tool must not show any permanent set, crack or sign of failure. |  |  |  |
| 8.0 | Certification |  |  |  |
| 8.1 | A copy of the last certificate awarded confirming the quality of the brand name and model of the tools offered should be attached to your bid. |  |  |  |
| 9.0 | Warranty: |  |  |  |
| 9.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment (FAO or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period. |  |  |  |

!!!FAILURE TO COMPLY WITH REQUIREMENTS STATED BELOW MAY RESULT IN YOUR BID BEING REJECTED!!!

1. You must duly complete all the empty spaces in the technical specification format to detail the specifications being offered.

2. Your bid must include supporting documentation requested in this form.

Company Name: ____________					Date: ___________________________

Official stamp and signature: __________________________________

Item 2 - Weeding Hoe

| # TECHNICAL SPECIFICATIONS REQUIRED |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |  |  |
| Item 2 |  | Description and purpose of the item | Make/ brand and model of item |  |  |  |
| Weeding hoe |  | Weeding hoe (maloda) with rectangular blade for weeding |  |  |  |  |
| Unit |  | Hoe head |  |  |  |  |
| Total quantity of units required |  | 51,990 Pieces | Name and address of the manufacturer of item |  |  |  |
| Country |  | Project code |  |  |  |  |
| Sudan |  | - 3,000 pieces Under FAO Project OSRO/SUD/809/ITA ; - 8,990 Under 2019 OFDA-Darfur ; - 24,000 pieces Under 2019 CERF ; - 5,500 Pieces Under 2019 UNTFHS Proposal ; - 9,000 pieces Under 2019 Korean Proposal ; - 1,500 pieces Under 2019 ECHO-Allait ; |  |  |  |  |
| 1.0. | Technical Specifications: General |  |  |  |  |  |
| 1.1 | Overall Length of Blade (mm): 270-280 |  |  |  |  |  |
| 1.2 | Length of Blade, excluding neck (mm): 170-190 |  |  |  |  |  |
| 1.3 | Blade Width (mm): 130-140 |  |  |  |  |  |
| 1.4 | Blade Thickness (mm): 2.5 min |  |  |  |  |  |
| 1.5 | Blade weight (kg): 0.5 - 0.7 |  |  |  |  |  |
| 1.6 | Length of the neck (mm): 45 - 50 |  |  |  |  |  |
| 1.7 | Diameter of the neck (mm): 30 -35 |  |  |  |  |  |
| 1.8 | Without handle |  |  |  |  |  |
| 1.9 | Type of cutting edge: Plain |  |  |  |  |  |
| 2.0 | Technical Specifications: Material specifications |  | IMPORTANT: FAO reserves the right to apply penalties after delivery if the lab test on material specifications show results that differ from what the bidder confirmed to comply |  |  |  |
|  | Blade: The steel for agricultural hoes shall be a medium to high carbon steel within the specified ranges given below. |  |  |  |  |  |
| 2.1 | Carbon : 0.40 – 0.80 % |  |  |  |  |  |
| 2.2 | Manganese : 0.50 – 0.90 % |  |  |  |  |  |
| 2.3 | Silicon : 0.35 % max. |  |  |  |  |  |
| 2.4 | Phosphorous : 0.06 % max. |  |  |  |  |  |
| 2.5 | Sulfur: 0.06 % max. |  |  |  |  |  |
| 3.0 | Technical Specifications: Heat treatment and hardness |  |  |  |  |  |
| 3.1 | The head and teeth will have a hardness of at least Rockwell C 22. |  |  |  |  |  |
| 4.0 | Technical Specifications: Construction |  |  |  |  |  |
| 4.1 | Blade: The hoe blade to be forged from one piece and the blade to be ground sharp on the cutting edge. |  |  |  |  |  |
| 4.2 | Blade: The forging must be symmetrical and free from flaws. All fins and flashes must be dressed off. The eye must be smooth internally, uniformly tapered, and must lie centrally in the forging. |  |  |  |  |  |
| 5.0 | Technical Specifications: Marking |  |  |  |  |  |
|  | The forging should be clearly and indelibly marked with the following: |  |  |  |  |  |
| 5.1 | Manufacturers name and/or trademark. |  |  |  |  |  |
| 5.2 | The nominal weight or size of the head. |  |  |  |  |  |
| 5.3 | The pattent number. |  |  |  |  |  |
| 6.0 | Technical Specifications: Preservative Treatment |  |  |  |  |  |
| 6.1 | The heads to be varnished or painted all over |  |  |  |  |  |
| 6.2 | If they are to be shipped by sea, then they should also be greased or individually wrapped in a rust inhibiting covering. |  |  |  |  |  |
| 6.3 | They must also be packed in strong Wooden boxes of 24 pieces with metallic belt |  |  |  |  |  |
| 7.0 | ##### Technical Specifications: Strength testing |  | ##### IMPORTANT: FAO reserves the right to apply penalties if the strength tests conducted before load and after delivery show results that differ from what the bidder confirmed to comply |  |  |  |
|  | ##### In addition to meeting the above specifications, the tools shall also meet the following strength tests: |  | ##### State below whether the tools offered will be able to meet the Strength Tests listed. |  |  |  |
| 7.1 | Strength Test: The tool is fitted with a temporary handle made from a metal tube as shown in Figure 1. A load of 30 kg is applied in increments of 5 kg, suspended from the handle at a distance of 1000 mm from the eye (the handle may not necessarily be horizontal). On removal of the load, the tool must not show any permanent set, crack or sign of failure. |  |  |  |  |  |
| 8.0 | Certification: |  |  |  |  |  |
| 8.1 | A copy of the last certificate awarded confirming the quality of the brand name and model of the tools offered should be attached to your bid. |  |  |  |  |  |
| 9.0 | Warranty: |  |  |  |  |  |
| 9.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment (FAO or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period. |  |  |  |  |  |

Quantity and Delivery of Weeding hoe (Hashasha)

- Delivery date to final destination: 15 May 2019

Figure 1 – Strength test

!!!FAILURE TO COMPLY WITH REQUIREMENTS STATED BELOW MAY RESULT IN YOUR BID BEING REJECTED!!!

1. You must duly complete all the empty spaces in the technical specification format to detail the specifications being offered.

2. Your bid must include supporting documentation requested in this form.

Company Name: ____________					Date: ___________________________

Official stamp and signature: __________________________________

Item 3 - Rake

| # TECHNICAL SPECIFICATIONS REQUIRED |  |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |  |  |
| Item 3 |  | Description and purpose of the item |  | Make/ brand and model of item |  |  |  |
| Rake without handle |  | Forged rake head without handle. |  |  |  |  |  |
| Unit |  | Rake head |  |  |  |  |  |
| Total quantity of units required |  | 42,575 Pieces |  | Name and address of the manufacturer of item |  |  |  |
| Country |  | Project code |  |  |  |  |  |
| Sudan |  | - 2,575 pieces Under 2019 OFDA Darfur ; - 9,000 pieces Under 2019 Korean Proposal ; - 24,000 pieces Under 2019 CERF Proposal ; - 5,500 pieces Under 2019 UNTFHS Proposal ; - 1,500 pieces Under 2019 ECHO-Allait |  |  |  |  |  |
| 1.0. | Technical Specifications: General |  |  |  |  |  |  |
| 1.1 | Width (mm): 300 - 350 |  |  |  |  |  |  |
| 1.2 | Number of teeth: 12 - 16 |  |  |  |  |  |  |
| 1.3 | Length of the teeth (mm): 70-100 |  |  |  |  |  |  |
| 1.4 | Thickness of the teeth (mm): min. 3 |  |  |  |  |  |  |
| 1.5 | Diameter of the eye for the handle (mm): 25 – 30 |  |  |  |  |  |  |
| 1.6 | Length of the socket (mm): min. 75 |  |  |  |  |  |  |
| 1.7 | Weight of head without handle (kg): 1.0 to 1.5 |  |  |  |  |  |  |
| 1.8 | Without handle |  |  |  |  |  |  |
| 2.0 | Technical Specifications: Material specifications |  |  | IMPORTANT: FAO reserves the right to apply penalties after delivery if the lab test on material specifications show results that differ from what the bidder confirmed to comply |  |  |  |
|  | Blade: The steel for agricultural hoes shall be a medium to high carbon steel within the specified ranges given below. |  |  |  |  |  |  |
| 2.1 | Carbon : 0.40 – 0.80 % |  |  |  |  |  |  |
| 2.2 | Manganese : 0.50 – 0.90 % |  |  |  |  |  |  |
| 2.3 | Silicon : 0.35 % max. |  |  |  |  |  |  |
| 2.4 | Phosphorous : 0.06 % max. |  |  |  |  |  |  |
| 2.5 | Sulfur: 0.06 % max. |  |  |  |  |  |  |
| 3.0 | Technical Specifications: Heat treatment and hardness |  |  |  |  |  |  |
| 3.1 | The head and teeth will have a hardness of at least Rockwell C 22. |  |  |  |  |  |  |
| 4.0 | Technical Specifications: Construction |  |  |  |  |  |  |
| 4.1 | Teeth may be curved or straight. The distance between teeth should be approximately 30 mm. |  |  |  |  |  |  |
| 4.2 | The head and teeth shall be forged from a single piece of steel |  |  |  |  |  |  |
| 4.3 | Forged steel construction ensure maximum strength to resist breaking and bending |  |  |  |  |  |  |
| 4.4 | The socket must be welded on the rear face of the head or made of a single piece of steel with the head. A socket fixed on the head through rivets will not be accepted. |  |  |  |  |  |  |
| 5.0 | Technical Specifications: Marking |  |  |  |  |  |  |
|  | The forging should be clearly and indelibly marked with the following: |  |  |  |  |  |  |
| 5.1 | Manufacturers name and/or trademark. |  |  |  |  |  |  |
| 5.2 | The nominal weight or size of the head. |  |  |  |  |  |  |
| 5.3 | The pattern number. |  |  |  |  |  |  |
| 6.0 | Technical Specifications: Preservative Treatment |  |  |  |  |  |  |
| 6.1 | The heads to be varnished or painted all over |  |  |  |  |  |  |
| 6.2 | If they are to be shipped by sea, then they should also be greased or individually wrapped in a rust inhibiting covering. |  |  |  |  |  |  |
| 6.3 | They must also be packed in strong Wooden boxes of 24 pieces with metallic belt |  |  |  |  |  |  |
| 7.0 | ##### Technical Specifications: Strength testing |  |  | ##### IMPORTANT: FAO reserves the right to apply penalties if the strength tests conducted before load and after delivery show results that differ from what the bidder confirmed to comply |  |  |  |
|  | ##### In addition to meeting the above specifications, the tools shall also meet the following strength tests: |  |  | ##### State below whether the tools offered will be able to meet the Strength Tests listed. |  |  |  |
| 7.1 | Strength Test: Using a standard wooden handle, the rake should be clamped in a vertical position with the rake head resting on the edge of a flat surface. A load of 35kg should be suspended from the centre of any spike (tooth) and maintained for two minutes. The spike tested must not show any deformation or permanent set after removal of the load. |  |  |  |  |  |  |
| 7.2 | Impact Test: Referring to the illustration below and using a standard hardwood handle, the rake handle should be attached to a wooden base so that when resting on a flat surface, the handle is horizontal. The head should then be raised to a height of one metre, pivoting about the base. The head should then be dropped so that the spikes fall onto a horizontal steel plate. This must be repeated four times, after which the spikes must not flatten or chip and no damage must be suffered by the rake head. |  |  |  |  |  |  |
| 8.0 | Certification |  |  |  |  |  |  |
| 8.1 | A copy of the last certificate awarded confirming the quality of the brand name and model of the tools offered should be attached to your bid. |  |  |  |  |  |  |
| 9.0 | Warranty: |  |  |  |  |  |  |
| 9.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment (FAO or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period. |  |  |  |  |  |  |

!!!FAILURE TO COMPLY WITH REQUIREMENTS STATED BELOW MAY RESULT IN YOUR BID BEING REJECTED!!!

1. You must duly complete all the empty spaces in the technical specification format to detail the specifications being offered.

2. Your bid must include supporting documentation requested in this form.

Company Name: ____________					Date: ___________________________

Official stamp and signature: __________________________________

Item 4 - sickle

| # TECHNICAL SPECIFICATIONS REQUIRED |  |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |
| Item 4 |  | Description and purpose of the item |  | Make/ brand and model of item |  |
| Sickle with handle |  | Sickle with hardwood handle for harvesting grain crops. |  |  |  |
| Unit |  | Sickle with handle |  |  |  |
| Total quantity of units required |  | 9000 |  | Name and address of the manufacturer of item |  |
| Country |  | Project code |  |  |  |
| Sudan |  |  |  |  |  |
| 1.0. | Technical Specifications: General |  |  |  |  |
| 1.1 | Overall Length (mm): 180 – 200 |  |  |  |  |
| 1.2 | Overall Width (mm): 170 - 180 |  |  |  |  |
| 1.3 | Blade Width (mm): 20 |  |  |  |  |
| 1.4 | Blade Thickness (mm): 2.0 |  |  |  |  |
| 1.5 | Handle Length (mm): 120 – 150 |  |  |  |  |
| 1.6 | Shape of Handle: Round |  |  |  |  |
| 1.7 | Diameter of handle (mm): 30 |  |  |  |  |
| 1.8 | Total Weight (kg): approx. 1.0 |  |  |  |  |
| 2.0 | Technical Specifications: Material specifications |  |  | IMPORTANT: FAO reserves the right to apply penalties after delivery if the lab test on material specifications show results that differ from what the bidder confirmed to comply |  |
|  | The blade should be made of high carbon steel within the specified range below: |  |  |  |  |
|  | Constituent |  | Specified range (%) |  |  |
| 2.1 | Carbon |  | 0.5 – 0.65 |  |  |
| 2.2 | Manganese |  | 0.5 – 0.80 |  |  |
| 2.3 | Silicon |  | 0.35 max |  |  |
| 2.4 | Phosphorous |  | 0.06 max |  |  |
| 2.5 | Sulfur |  | 0.06 max |  |  |
| 3.0 | Technical Specifications: Heat treatment and hardness |  |  |  |  |
| 3.1 | After forging, the blade should be normalised and then hardened and tempered to give Rockwell C 45/50 along the whole length of the blade. |  |  |  |  |
| 4.0 | Technical Specifications: Construction |  |  |  |  |
| 4.1 | Blades: Serrated edges to be ground sharp. |  |  |  |  |
| 4.2 | Handles: The blades should be firmly fixed to handles in a way that does not allow the blade to rotate loosely or slip off after sustained use. Either countersunk rivets or screws can be used, or the tang from the blade should run through the whole length of the handle and emerge at the other end of the handle and be bent backwards, to ensure that the handle cannot be pulled off the blade. It is important that this is done in such as way that the protruding metal is not exposed to the operator’s hand. If the bent tang does not also prevent the handle from rotating then the handle should be positively located onto the blade by countersunk rivet or screw. |  |  |  |  |
| 5.0 | ##### Technical Specifications: Handle |  |  |  |  |
| 5.1 | The handle should be made of a suitable hardwood with a specific gravity of 0.66 to 0.80 after seasoning to not more than 20% moisture content. |  |  |  |  |
| 5.2 | Length of handle (mm): 1200 app |  |  |  |  |
| 5.3 | Diameter of handle (mm): to suit dimensions of fitting eye. |  |  |  |  |
| 6.0 | Marking |  |  |  |  |
|  | The forging should be clearly and indelibly marked with the following: |  |  |  |  |
| 6.1 | Manufacturers name and/or trademark. |  |  |  |  |
| 6.2 | The pattern number. |  |  |  |  |
| 7.0 | Preservative treatment: |  |  |  |  |
| 7.1 | The blade to be varnished all over and the handles to be oiled. |  |  |  |  |
| 7.2 | If they are to be shipped by sea, they should also be greased or individually wrapped in a rust inhibiting covering. |  |  |  |  |
| 7.3 | They should also be packed in a strong cardboard box for shipment |  |  |  |  |
| 8.0 | Technical Specifications: Strength testing |  |  | IMPORTANT: FAO reserves the right to apply penalties if the strength tests conducted before load and after delivery show results that differ from what the bidder confirmed to comply |  |
|  | In addition to meeting the above specifications, the tools shall also meet the following strength tests: |  |  | State below whether the tools offered will be able to meet the Strength Tests listed. |  |
| 8.1 | Strength Test: With the handle clamped horizontally and with the blade also in a horizontal plane, a load of 5 kg shall be gradually applied to the blade tip and maintained for 2 minutes. The blade shall then be turned over and the test repeated until a total of four tests has been carried out. On final removal of the load, the tool shall show no signs of damage to the head or loosening of the handle, nor shall there be any permanent set in excess of 5 mm measured at the end of the blade. |  |  |  |  |
| 9.0 | Certification |  |  |  |  |
| 9.1 | A copy of the last certificate awarded confirming the quality of the brand name and model of the tools offered should be attached to your bid. |  |  |  |  |
| 10.0 | Warranty: |  |  |  |  |
| 10.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment (FAO or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period. |  |  |  |  |

!!!FAILURE TO COMPLY WITH REQUIREMENTS STATED BELOW MAY RESULT IN YOUR BID BEING REJECTED!!!

1. You must duly complete all the empty spaces in the technical specification format to detail the specifications being offered.

2. Your bid must include supporting documentation requested in this form.

Company Name: ____________					Date: ___________________________

Official stamp and signature: __________________________________

Item 5 - DAP mouldboard donkey plough

| # TECHNICAL SPECIFICATIONS REQUIRED |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |
| --- | --- | --- | --- | --- |
|  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |
| Item 5 |  | Description and purpose of the item | Make/ brand and model of item |  |
| DAP mouldboard donkey plough |  | DAP mouldboard plough suitable for use with donkey, a single animal. Savannah type or similar. |  |  |
| Unit |  | Plough + spare parts + accessories |  |  |
| Total quantity of units required |  | 6,683 pieces | Name and address of the manufacturer of item |  |
| Country |  | Project code |  |  |
| Sudan |  | - 683 pieces Under 2019 OFDA Darfur-Nyala; - 4,200 pieces Under 2019 CERF-Nyala; - 1,800 pieces Under 2019 CERF-El Geneina; |  |  |
| 1.0. | Technical Specifications: General |  |  |  |
| 1.1 | Single furrow mouldboard plough, suitable for use with a single donkey |  |  |  |
| 1.2 | Furrow width (perpendicular distance from the landside to the rear part of the share): 200 mm |  |  |  |
| 1.3 | Working depth up to 130 mm |  |  |  |
| 1.4 | Mouldboard shape: adapted for use in multiple soil conditions, with good inversion and weed covering properties |  |  |  |
| 1.5 | Mouldboard: minimum thickness 2 mm |  |  |  |
| 1.6 | Share: flat or upset |  |  |  |
| 1.7 | Share: minimum: 8 mm thickness |  |  |  |
| 1.8 | Share material: medium to high carbon steel and heat treated to a hardness of minimum HRC 46 |  |  |  |
| 1.9 | Frog: thickness 4.5 - 5 mm |  |  |  |
| 1.10 | Landside: minimum 6 mm thickness |  |  |  |
| 1.11 | Beam: I beam with thickness of I, 10 mm and width 20 mm |  |  |  |
| 1.12 | Hitch assembly including 3 holes on the front part of the beam |  |  |  |
| 1.13 | Beam length: 800 to 830 mm |  |  |  |
| 1.14 | Vertical handle assembly approx. 1000 mm long (approx. 700mm above the beam) connecting the landside to the beam. |  |  |  |
| 1.15 | Sub-frame bolted to the main beam. |  |  |  |
| 1.16 | Total weight of the plough assembly should not exceed 30 kg. |  |  |  |
| 2.0 | Technical Specifications: Construction |  |  |  |
| 2.1 | Beam and handles made of steel. The handles must be completed to a smooth finish. |  |  |  |
| 2.2 | All metallic parts must be free from flaws. All fins and flashes must be dressed off. |  |  |  |
| 2.3 | The beam and handles must be varnished or painted all over. |  |  |  |
| 3.0 | Technical Specifications: Marking |  |  |  |
|  | The plough must be clearly and indelibly marked with the following: |  |  |  |
| 3.1 | Manufacturer’s name and / or trademark |  |  |  |
| 3.2 | Plough model |  |  |  |
| 3.3 | FAO logo with project code |  |  |  |
| 3.4 | Donor logo |  |  |  |
| 4.0 | Accessories |  |  |  |
| 4.1 | 1 complete set of spanners for each plough for undertaking routine maintenance and adjustment. |  |  |  |
| 4.2 | 1 Trek Chain 3.5 metres in length. |  |  |  |
| 4.3 | ##### 1 Yoke U bolt of approx. diameter 14mm + 2 nuts to suit. |  |  |  |
| 5.0 | Certification: |  |  |  |
| 5.1 | A copy of the last certificate awarded confirming the quality of the brand name and model of the ploughs offered should be attached to your bid. |  |  |  |
| 6.0 | Manuals: |  |  |  |
| 6.1 | Operators hand book (printed or in CD) in English or Arabic, one for each unit. |  |  |  |
| 6.2 | The cost of the manuals should be included in the DAP/DAT cost of the equipment |  |  |  |
| 7.0 | Spare Parts: |  |  |  |
| 7.1 | A list of manufacturer’s recommended spare parts, sufficient for 2 years of operation must be submitted for each unit. The list must be itemised, costed and submitted with your bid. The list must include at least a share, a landside, an assortment of nuts and bolts and other fast wearing parts. |  |  |  |
| 8.0 | After sales service: |  |  |  |
| 8.1 | Supply the name, Tel./Fax number and email or postal address of your service agents in Sudan or neighbouring countries. |  |  |  |
| 8.2 | State who will be responsible for undertaking the pre-delivery inspection of your equipment upon arrival. |  |  |  |
| 8.3 | The bid price must include the cost of delivery and installation |  |  |  |
| 9.0 | Warranty: |  |  |  |
| 9.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment (FAO or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period. |  |  |  |

!!!FAILURE TO COMPLY WITH REQUIREMENTS STATED BELOW MAY RESULT IN YOUR BID BEING REJECTED!!!

1. You must duly complete all the empty spaces in the technical specification format to detail the specifications being offered.

2. Your bid must include supporting documentation requested in this form.

Company Name: ____________		Date: ___________________________Official stamp and signature: __________________________________

Item 6 - Donkey cart

| # TECHNICAL SPECIFICATIONS REQUIRED |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |
| --- | --- | --- | --- | --- |
|  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |
| Item 6 |  | Description and purpose of the item | Make/ brand and model of item |  |
| Donkey cart |  | Two wheeled cart to be pulled by donkey traction with tray for transport of goods or persons in rural areas. |  |  |
| Unit |  | Cart + spare parts |  |  |
| Total quantity of units required |  | 284 | Name and address of the manufacturer of item |  |
| Country |  | Project code |  |  |
| Sudan |  | 150 under 2019 OFDA Darfur and 134 under 2019 ECHO-Allait |  |  |
| 1.0. | Technical Specifications: Axle |  |  |  |
| 1.1 | Number of wheels: Two |  |  |  |
| 1.2 | Tyre size: 135 x 15 |  |  |  |
| 1.3 | Wheel base (width): 125 to 135 cm |  |  |  |
| 1.4 | Cart height with wheels: 85 to 95 cm |  |  |  |
| 1.5 | Toeing bar: Metallic Pipe tubing |  |  |  |
| 1.6 | Axle assembly: With roller bearing |  |  |  |
| 1.7 | Equipped with two metal coupling arms /support bars made with metal tubing: 70 mm diameter; length: 1.2 m app.; thickness: 3.5 mm app. |  |  |  |
| 2.0 | Technical Specifications: Tray / Platform |  |  |  |
| 2.1 | Tray surrounded by a frame constructed from metal angles |  |  |  |
| 2.2 | Tray dimensions: Width: 110 to 120 cm; Length: 150 to 160 cm |  |  |  |
| 2.3 | Equipped with side flakes: Yes |  |  |  |
| 2.4 | Flakes height: 50 cm approximately |  |  |  |
| 2.5 | Tray supports reinforced with welded plates braced to the tray by fixing plates |  |  |  |
| 2.6 | Tray topping material: Red wood, 15 to 25 mm thickness |  |  |  |
| 2.7 | Tray frame material: Metallic angles (40 x 40 x 4 mm) |  |  |  |
| 3.0 | Spare parts: |  |  |  |
| 3.1 | A list of manufacturer’s recommended spare parts, sufficient for 2 years of operation must be submitted for each unit. The list must be itemised, costed and submitted with your bid. |  |  |  |
| 4.0 | After sales service: |  |  |  |
| 4.1 | Supply the name, Tel./Fax number and email or postal address of your service agents in Sudan |  |  |  |
| 4.2 | State who will be responsible for undertaking the pre-delivery inspection of your equipment upon arrival. |  |  |  |
| 4.3 | The bid price must include the cost of delivery and installation (or delivery of fully assembled item). |  |  |  |
| 5.0 | Warranty: |  |  |  |
| 5.1 | A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment (FAO or end-user) must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period. |  |  |  |

CART´S FRAME DESIGN:

IMAGE OF CART (show purpose only):

!!!FAILURE TO COMPLY WITH REQUIREMENTS STATED BELOW MAY RESULT IN YOUR BID BEING REJECTED!!!

1. You must duly complete all the empty spaces in the technical specification format to detail the specifications being offered.

2. Your bid must include supporting documentation requested in this form.

Company Name: ____________		Date: ___________________________

Official stamp and signature: __________________________________

1 – Delivery plan

| # | Item description | Delivery location | Unit | Quantity | Required delivery time | Offered best delivery time in calendar days after issuance of PO (to be filled in by bidder) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Digging Hoe | Port Sudan | each | 3000 | 30 calendar days after issuance of PO (no later than 30 May 2019) |  |
|  |  | Nyala | each | 15415 |  |  |
|  |  | EL Genaina | each | 10575 |  |  |
|  |  | Zalinge | each | 3000 |  |  |
|  |  | El Fashir | each | 1500 |  |  |
|  |  | Damazine | each | 9000 |  |  |
|  |  | Kadugli | each | 5500 |  |  |
|  |  | EL Fula | each | 4000 |  |  |
|  |  | Total |  | 51 990 |  |  |
| 2 | Weeding Hoe | Port Sudan | each | 3000 |  |  |
|  |  | Nyala | each | 15415 |  |  |
|  |  | EL Genaina | each | 10575 |  |  |
|  |  | Zalinge | each | 3000 |  |  |
|  |  | El Fashir | each | 1500 |  |  |
|  |  | Damazine | each | 9000 |  |  |
|  |  | Kadugli | each | 5500 |  |  |
|  |  | EL Fula | each | 4000 |  |  |
|  |  | Total |  | 51 990 |  |  |
| 3 | Rake | El Fashir | each | 1500 |  |  |
|  |  | EL Genaina | each | 10575 |  |  |
|  |  | Kadugli | each | 5500 |  |  |
|  |  | Damazine | each | 9000 |  |  |
|  |  | Nyala | each | 12000 |  |  |
|  |  | EL Fula | each | 4000 |  |  |
|  |  | Total |  | 42 575 |  |  |
| 4 | Sickle | Damazine | each | 9000 |  |  |
|  |  | Total |  | 9 000 |  |  |
| 5 | Donkey Plough | Nyala | each | 4883 |  |  |
|  |  | EL Genaina | each | 1800 |  |  |
|  |  | Total |  | 6 683 |  |  |
| 6 | Donkey Carts | Nyala | each | 75 |  |  |
|  |  | EL Genaina | each | 75 |  |  |
|  |  | El Fashir | each | 134 |  |  |
|  |  | Total |  | 284 |  |  |

Company Name: ____________		Date: ___________________________

Official stamp and signature: __________________________________

2 – Stock and Logistic Capacity

FAO reserves the right to arrange for pre-shipment inspection within the first week of the issuance of the PO, to check on the progress of the PO implementation and in the event of unsatisfactory inspection results, FAO may terminate the Purchase Order with immediate effect and without any without any further obligations and liabilities for FAO.

Please confirm acceptance of a pre-shipment quality and quantity inspection and its terms and conditions described in the paragraph above:

YES                          NO

In addition FAO will carry out quality tests (including full laboratory analysis) and quantity inspection at loading point and unloading points.

    - STOCK AVAILABILITY

Does your company have a stock of the FAO required agricultural tools complying with the present requirements in all respects?

YES                           NO

If yes, indicate below the quantity in stock:

| No | Item description | Quantity required (each) | Quantity available in stock (each) (to be filled in by bidder) |
| --- | --- | --- | --- |
| 1 | Digging Hoe | 51 990 |  |
| 2 | Weeding Hoe | 51 990 |  |
| 3 | Rake | 42 575 |  |
| 4 | Sickle with handle | 9 000 |  |
| 5 | Donkey Ploughs | 6 683 |  |
| 6 | Donkey Carts | 284 |  |

FAO reserves the right to arrange for a pre-award verification of the manufacturing facility and quantities available in stock.

Warehouse 1 address and contact details:   .................................................................................................................

Warehouse 2 address and contact details:   .................................................................................................................

Warehouse 3 address and contact details:   .................................................................................................................

Company Name: ____________		Date: ___________________________

Official stamp and signature: __________________________________

    - LOGISTICS CAPACITIES

Provide here below details of your logistics plan to deliver the offered quantity of FAO required agricultural tools within the proposed delivery date(s) as indicated in your offer.

  - Quantity that can be delivered each week after receipt of Purchase Order:

| FAO requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | to be filled in by bidder |  |  |  |  |
| Description | Total quantity required (each) | Total quantity offered (each) | Quantity to be delivered within first week after PO signature | Quantity to be delivered within second week after PO signature | Quantity to be delivered within third week after PO signature | Quantity to be delivered within forth week after PO signature |
| Digging Hoe | 51 990 |  |  |  |  |  |
| Weeding Hoe | 51 990 |  |  |  |  |  |
| Rake | 42 575 |  |  |  |  |  |
| Sickle with handle | 9 000 |  |  |  |  |  |
| Donkey Ploughs | 6 683 |  |  |  |  |  |
| Donkey Carts | 284 |  |  |  |  |  |

In responding to our tender invitation and accepting FAO Terms and Conditions you hereby specifically authorize FAO and FAO nominated inspection company, to visit without formalities your premises and/ or warehouses and /or manufacturing facilities before or after issuing the eventual Order.

N.B.: Lack of compliance with this requirement may result in offer being disqualified.

Company Name: ________________________________		Date: ___________________________

Official stamp and signature: ________________________________

Item 1 – Digging Hoe

| Item | # | Delivery destination | Qty requested (in each) | Qty Offered (in each) | Unit Price FCA (in US$*) | Total Cost FCA (in US$*) | Freight Unit Cost (in US$*) | Freight Total Cost (in US$*) | Insurance Cost (in US$*) | Total Cost DAT (in US$*) | Best Delivery Time (specify no. of calendar days after PO issuance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (1) | (2) | (3)=(1)x(2) | (4) | (5)=(1)x(4) | (6) | (7) = (3)+(5)+(6) |  |
| Digging Hoe | 1 | Port Sudan | 3000 |  |  |  |  |  |  |  |  |
|  | 2 | Nyala | 15415 |  |  |  |  |  |  |  |  |
|  | 3 | EL Genaina | 10575 |  |  |  |  |  |  |  |  |
|  | 4 | Zalinge | 3000 |  |  |  |  |  |  |  |  |
|  | 5 | El Fashir | 1500 |  |  |  |  |  |  |  |  |
|  | 6 | Damazine | 9000 |  |  |  |  |  |  |  |  |
|  | 7 | Kadugli | 5500 |  |  |  |  |  |  |  |  |
|  | 8 | EL Fula | 4000 |  |  |  |  |  |  |  |  |
| TOTAL |  |  | 51 990 |  |  |  |  |  |  |  |  |

* If the quote is in a different currency than specified here, FAO will convert prices for the commercial evaluation to a single currency using the official UN exchange rate corresponding to the date of the offer.

NOTE regarding arithmetical errors and discrepancies:

- If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price will prevail and the total price will be corrected, unless in the opinion of evaluators, there is an obvious misplacement of the decimal point in the unit price, in which case the total price as quoted will govern and the unit price will be corrected.

- If there is an error in a total corresponding to the addition or subtraction of subtotals, the subtotals will prevail and the total will be corrected.

- In case of discrepancy between a word and figures, the amount in words will prevail, unless the amount expressed in words is related to an arithmetic error, in which case the amount in figures will prevail subject to the note above.

Company Name: ________________________________					Date: ___________________________

Official stamp and signature: ___________________________

Item 2 – Weeding Hoe

| Item | # | Delivery destination | Qty requested (in each) | Qty Offered (in each) | Unit Price FCA (in US$*) | Total Cost FCA (in US$*) | Freight Unit Cost (in US$*) | Freight Total Cost (in US$*) | Insurance Cost (in US$*) | Total Cost DAT (in US$*) | Best Delivery Time (specify no. of calendar days after PO issuance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (1) | (2) | (3)=(1)x(2) | (4) | (5)=(1)x(4) | (6) | (7) = (3)+(5)+(6) |  |
| Weeding Hoe | 1 | Port Sudan | 3000 |  |  |  |  |  |  |  |  |
|  | 2 | Nyala | 15415 |  |  |  |  |  |  |  |  |
|  | 3 | EL Genaina | 10575 |  |  |  |  |  |  |  |  |
|  | 4 | Zalinge | 3000 |  |  |  |  |  |  |  |  |
|  | 5 | El Fashir | 1500 |  |  |  |  |  |  |  |  |
|  | 6 | Damazine | 9000 |  |  |  |  |  |  |  |  |
|  | 7 | Kadugli | 5500 |  |  |  |  |  |  |  |  |
|  | 8 | EL Fula | 4000 |  |  |  |  |  |  |  |  |
| TOTAL |  |  | 51 990 |  |  |  |  |  |  |  |  |

* If the quote is in a different currency than specified here, FAO will convert prices for the commercial evaluation to a single currency using the official UN exchange rate corresponding to the date of the offer.

NOTE regarding arithmetical errors and discrepancies:

- If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price will prevail and the total price will be corrected, unless in the opinion of evaluators, there is an obvious misplacement of the decimal point in the unit price, in which case the total price as quoted will govern and the unit price will be corrected.

- If there is an error in a total corresponding to the addition or subtraction of subtotals, the subtotals will prevail and the total will be corrected.

- In case of discrepancy between a word and figures, the amount in words will prevail, unless the amount expressed in words is related to an arithmetic error, in which case the amount in figures will prevail subject to the note above.

Company Name: ________________________________					Date: ___________________________

Official stamp and signature: ___________________________

Item 3 – Rake

| Item | # | Delivery destination | Qty requested (in each) | Qty Offered (in each) | Unit Price FCA (in US$*) | Total Cost FCA (in US$*) | Freight Unit Cost (in US$*) | Freight Total Cost (in US$*) | Insurance Cost (in US$*) | Total Cost DAT (in US$*) | Best Delivery Time (specify no. of calendar days after PO issuance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (1) | (2) | (3)=(1)x(2) | (4) | (5)=(1)x(4) | (6) | (7) = (3)+(5)+(6) |  |
| Rake | 1 | El Fashir | 1500 |  |  |  |  |  |  |  |  |
|  | 2 | EL Genaina | 10575 |  |  |  |  |  |  |  |  |
|  | 3 | Kadugli | 5500 |  |  |  |  |  |  |  |  |
|  | 4 | Damazine | 9000 |  |  |  |  |  |  |  |  |
|  | 5 | Nyala | 12000 |  |  |  |  |  |  |  |  |
|  | 6 | EL Fula | 4000 |  |  |  |  |  |  |  |  |
| TOTAL |  |  | 42 575 |  |  |  |  |  |  |  |  |

* If the quote is in a different currency than specified here, FAO will convert prices for the commercial evaluation to a single currency using the official UN exchange rate corresponding to the date of the offer.

NOTE regarding arithmetical errors and discrepancies:

- If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price will prevail and the total price will be corrected, unless in the opinion of evaluators, there is an obvious misplacement of the decimal point in the unit price, in which case the total price as quoted will govern and the unit price will be corrected.

- If there is an error in a total corresponding to the addition or subtraction of subtotals, the subtotals will prevail and the total will be corrected.

- In case of discrepancy between a word and figures, the amount in words will prevail, unless the amount expressed in words is related to an arithmetic error, in which case the amount in figures will prevail subject to the note above.

Company Name: ________________________________					Date: ___________________________

Official stamp and signature: ___________________________

Item 4 – Sickle

| Item | # | Delivery destination | Qty requested (in each) | Qty Offered (in each) | Unit Price FCA (in US$*) | Total Cost FCA (in US$*) | Freight Unit Cost (in US$*) | Freight Total Cost (in US$*) | Insurance Cost (in US$*) | Total Cost DAT (in US$*) | Best Delivery Time (specify no. of calendar days after PO issuance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (1) | (2) | (3)=(1)x(2) | (4) | (5)=(1)x(4) | (6) | (7) = (3)+(5)+(6) |  |
| Sickle | 1 | Damazine | 9000 |  |  |  |  |  |  |  |  |
| TOTAL |  |  | 9 000 |  |  |  |  |  |  |  |  |

* If the quote is in a different currency than specified here, FAO will convert prices for the commercial evaluation to a single currency using the official UN exchange rate corresponding to the date of the offer.

NOTE regarding arithmetical errors and discrepancies:

- If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price will prevail and the total price will be corrected, unless in the opinion of evaluators, there is an obvious misplacement of the decimal point in the unit price, in which case the total price as quoted will govern and the unit price will be corrected.

- If there is an error in a total corresponding to the addition or subtraction of subtotals, the subtotals will prevail and the total will be corrected.

- In case of discrepancy between a word and figures, the amount in words will prevail, unless the amount expressed in words is related to an arithmetic error, in which case the amount in figures will prevail subject to the note above.

Company Name: ________________________________					Date: ___________________________

Official stamp and signature: ___________________________

Item 5 – Donkey Plough

| Item | # | Delivery destination | Qty requested (in each) | Qty Offered (in each) | Unit Price FCA (in US$*) | Total Cost FCA (in US$*) | Freight Unit Cost (in US$*) | Freight Total Cost (in US$*) | Insurance Cost (in US$*) | Total Cost DAT (in US$*) | Best Delivery Time (specify no. of calendar days after PO issuance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (1) | (2) | (3)=(1)x(2) | (4) | (5)=(1)x(4) | (6) | (7) = (3)+(5)+(6) |  |
| Donkey Plough | 1 | Nyala | 4883 |  |  |  |  |  |  |  |  |
|  | 2 | EL Genaina | 1800 |  |  |  |  |  |  |  |  |
| TOTAL |  |  | 6 683 |  |  |  |  |  |  |  |  |

* If the quote is in a different currency than specified here, FAO will convert prices for the commercial evaluation to a single currency using the official UN exchange rate corresponding to the date of the offer.

NOTE regarding arithmetical errors and discrepancies:

- If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price will prevail and the total price will be corrected, unless in the opinion of evaluators, there is an obvious misplacement of the decimal point in the unit price, in which case the total price as quoted will govern and the unit price will be corrected.

- If there is an error in a total corresponding to the addition or subtraction of subtotals, the subtotals will prevail and the total will be corrected.

- In case of discrepancy between a word and figures, the amount in words will prevail, unless the amount expressed in words is related to an arithmetic error, in which case the amount in figures will prevail subject to the note above.

Company Name: ________________________________					Date: ___________________________

Official stamp and signature: ___________________________

Item 6 – Donkey Cart

| Item | # | Delivery destination | Qty requested (in each) | Qty Offered (in each) | Unit Price FCA (in US$*) | Total Cost FCA (in US$*) | Freight Unit Cost (in US$*) | Freight Total Cost (in US$*) | Insurance Cost (in US$*) | Total Cost DAT (in US$*) | Best Delivery Time (specify no. of calendar days after PO issuance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (1) | (2) | (3)=(1)x(2) | (4) | (5)=(1)x(4) | (6) | (7) = (3)+(5)+(6) |  |
| Donkey Cart | 1 | Nyala | 75 |  |  |  |  |  |  |  |  |
|  | 2 | EL Genaina | 75 |  |  |  |  |  |  |  |  |
|  | 3 | EL Fashir | 134 |  |  |  |  |  |  |  |  |
| TOTAL |  |  | 284 |  |  |  |  |  |  |  |  |

* If the quote is in a different currency than specified here, FAO will convert prices for the commercial evaluation to a single currency using the official UN exchange rate corresponding to the date of the offer.

NOTE regarding arithmetical errors and discrepancies:

- If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price will prevail and the total price will be corrected, unless in the opinion of evaluators, there is an obvious misplacement of the decimal point in the unit price, in which case the total price as quoted will govern and the unit price will be corrected.

- If there is an error in a total corresponding to the addition or subtraction of subtotals, the subtotals will prevail and the total will be corrected.

- In case of discrepancy between a word and figures, the amount in words will prevail, unless the amount expressed in words is related to an arithmetic error, in which case the amount in figures will prevail subject to the note above.

Company Name: ________________________________					Date: ___________________________

Official stamp and signature: ______________________