Specifications
Thermodynamic cycle Diesel 4 stroke
Air intake TAA
Arrangement 4L
Bore x Stroke mm 95.8 x 104
Total displacement l 2.998
Valves per cylinder n. 4
Cooling system liquid
Direction of rotation (viewed facing flywheel) CCW
Engine management electronic
Injection system Electronic Common Rail (E.C.R.)
Electrical system
Voltage V 12
Standard configuration
Flywheel housing type SAE 4
Flywheel size inch 8
Air filter rear side
Turbocharger Waste Gate (water cooled)
Turbo with Aftercooler (TAA)
Heat exchanger tube type
Exhaust gas - water mixer -
Water charge tank included
Fuel filter n. 1
Fuel prefilter included (loose)
Fuel pump included (loose)
Oil filter n. 1
Oil sump aluminium
Oil vapours blow-by circuit front
Oil heat exchanger built in the crankcase
Oil filler on front cover
Starting motor 12V - 2.3kW
Alternator 12V - 110A
Engine stop device by electronic central unit
Wiring harness with EDC (Electronic Diesel Control)
Painting color white “ICE”
Not included in the standard configuration
Battery - minimum capacity recommended 110 Ah
Battery - minimum cold cranking capacity recommended 800 A

如果没有这个型号 其他的依维柯牌子的也行

如果没有依维柯这个牌子 Man 或者 Marcides 牌子也可以。