Section 5a: Technical Specifications

| # | Item to be supplied Description/Specifications | Qty | Delivery Date |
| --- | --- | --- | --- |
| 1 | STEEL BUNK BEDS - Triple bunk bed (three sleeping spaces at different levels), made of steel for use in prison cells - First level shall be 0.25 metres from the base of the bed. - Second level shall be at 1.2 metres from the base of the bed - Third level shall be at 2.0 metres from the base of the bed - The feet of the bed shall have a flat plate of 140 mm by 140 mm by 6.0 mm to enable its mounting to the floor with bolts of size 17. - Made from a main frame of 40 mm by 40 mm by 1.6 mm mild steel square tubes - 2.0 metres by 1.0 metres to enable accommodating single mattresses of 1.89 metres by 0.91 metres at each level. - The mattress support shall be 6 mm thick plywood on a support frame of 30 mm by 5 mm thick mild steel flat bars in both longitudinal and horizontally across the bed - Each sleeping level of the bed should be dismountable. - The bunk bed should provide for a climbing-stairs to the upper two levels and be mounted towards the longitudinal end of the bunk bed. - Bunk beds should be designed to provide safe usage - The steel for bunk beds should be compliant with Malawi Bureau of Standards - The bunk beds should be painted with Red oxide as first coat, then then grey as a final colour | 350 | Six weeks after signing the order |
| 2 | MATTRESSES - The mattress shall be 1.89 metres by 0.91 metres and 102.4 mm (4 inches). - The mattress shall be covered by a PVC Re-enforced fabric | 1050 | Six weeks after signing the order |

Clustering items by lots, if any, is recommended, especially if partial bids will be allowed.